## Router

enable

configure terminal

VLAN 10

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.1.1 255.255.255.192

VLAN 20

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.1.65 255.255.255.192

VLAN 30

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.1.129 255.255.255.192

VLAN 40

interface g0/0.40

encapsulation dot1Q 40

ip address 192.168.1.193 255.255.255.192

Make sure to set the Gigabyte 0/0 interface up without any ip address.

## Switch 1

! Assign ports for devices

interface range fa0/1-3

switchport mode access

switchport access vlan 10

interface range fa0/4-6

switchport mode access

switchport access vlan 20

interface gig0/1

switchport mode trunk

interface gig0/0

switchport mode trunk

## Switch 2

! Assign ports for devices

interface range fa0/1-3

switchport mode access

switchport access vlan 30

interface range fa0/4-6

switchport mode access

switchport access vlan 40

interface gig0/1

switchport mode trunk

## PICTURES